CSC-203L Computer Network

Project Report

Multi-Campus University Network

Submitted by:

Abdul Moez		2024-SE-14

Umair Arshad		2024-SE-38

M. Zeeshan		2024-SE-13

Submitted to:

Sir Nouman

Dated: 9th January 2026

Department of Computer Science

University of Engineering and Technology Lahore, New Campus

Table of Contents

1.	Abstract	3

2.	Introduction	3

3.	Devices Used	4

4.	Network Diagram	5

5.	VLAN And IP Addressing Table	5

5.1.	Main Campus	5

5.1.1.	CS Department	5

5.1.2.	Electrical Department	6

5.1.3.	Mechanical Department	7

5.2.	New Campus	8

5.2.1.	CS Department	8

5.2.2.	Electrical Department	8

5.2.3.	Mechanical Department	9

5.3.	FSD Campus	10

5.3.1.	CS Department	10

5.3.2.	Electrical Department	10

5.3.3.	Mechanical Department	11

6.	Key Configuration	12

6.1.	Switch Configuration	12

6.2.	Router Configuration	12

6.2.1.	Internal Routing	12

6.3.	Router CLI	12

6.4.	Server Configuration	13

7.	Testing Results	14

7.1.	Pinging	14

7.1.1.	Ping Test (CS → EE)	14

7.2.	Trace Route Trace route from one department to another department	14

8.	Packet Simulation	15

9.	Troubleshooting Commands Used	15

10.	Redundancy And Failover Testing	15

11.	Conclusion	15

# Abstract

This project presents the design and implementation of a scalable, high-availability university network across multiple campus locations using Cisco Packet Tracer. The architecture features a central main campus and two sub-campuses, utilizing a star-shaped core topology for central connectivity while maintaining inter-campus communication via border routers. Within each sub-campus, the internal infrastructure is optimized using Layer 3 switches to handle inter-VLAN routing and local traffic, which are then linked to internal routers for departmental communication. To ensure efficient traffic management and security, the design employs VLANs for departmental isolation (Computer Science, Electrical Engineering, and Administration) and VLSM for optimized IPv4 addressing. Routing reliability is achieved through a hybrid protocol approach: OSPF manages internal campus traffic, while BGP is implemented on border routers to provide load balancing and redundancy for inter-campus and ISP connectivity. The final implementation is validated through comprehensive testing of end-to-end connectivity, server accessibility, and Access Control List (ACL) verification.

# Introduction

Modern university environments require sophisticated network infrastructures that can seamlessly connect diverse departments across geographically dispersed sub-campuses while maintaining high performance and security. This project focuses on designing a multi-campus, requiring the implementation of a complex system that eliminates single points of failure.

The network design is structured around several critical architectural components:

- Hierarchical Topology: The main campus acts as a central hub, connecting to sub-campuses via a star-shaped design with border routers at each location to facilitate inter-campus communication and ISP connectivity.

- Internal Sub-Campus Architecture: Each sub-campus utilizes Layer 3 switches to perform high-speed inter-VLAN routing between departments. These switches connect to internal routers that handle routing between specific departmental units and the wider campus network.

- Departmental Segmentation: To optimize performance and security, departments such as Computer Science (VLAN 10), Electrical Engineering (VLAN 20), and Administration (VLAN 30) are isolated into separate VLANs.

- Efficient IP Management: The design uses VLSM (Variable Length Subnet Masking) within the 192.168.0.0/16 private IP range to ensure efficient address allocation across all subnets.

- Core Services: Each sub-campus supports essential services including Web (HTTP), Email, DNS, and FTP servers, with access controlled by ACLs to restrict or allow traffic based on departmental requirements.

By utilizing Cisco Packet Tracer, this project simulates a real-world enterprise environment. It provides a platform to verify routing functionality via OSPF and BGP, ensuring the network can handle failover scenarios and maintain connectivity between all departmental units across the university.

# Devices Used

| Device Type | Model Number | Where to Find |
| --- | --- | --- |
| Border Router | 2911 | Routers → 2911 |
| Internal Router | 2911 | Routers →2911 |
| Layer 3 Switch | 3650-24PS | Switches → 3650-24PS |
| Access Switch | 2960-24TT | Switches → 2960-24TT |

# Network Diagram

# VLAN And IP Addressing Table

## Main Campus

### CS Department

| Network Segment | Hosts Required | Hosts Available | Network Address | Subnet Mask | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VLAN 10 Admin | 50 | 62 (/26) | 192.168.1.0 | 255.255.255.192 | 192.168.1.1 | 192.168.1.62 | 192.168.1.63 |
| VLAN 20 Lab 2 & VLAN 20 Lab 1 | 40 | 62 (/26) | 192.168.1.64 | 255.255.255.192 | 192.168.1.65 | 192.168.1.126 | 192.168.1.127 |
| VLAN 30 | 35 | 62 (/26) | 192.168.1.128 | 255.255.255.192 | 192.168.1.129 | 192.168.1.198 | 192.168.1.191 |
| R4 to Main Campus Router Serial 0/0/0 | 2 | 2 (/30) | 192.168.100.0 | 255.255.255.252 | 192.168.100.1 | 192.168.100.2 | 192.168.100.3 |

### Electrical Department

| Network Segment | Hosts Required | Hosts Available | Network Address | Subnet Mask | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VLAN 10 Admin | 50 | 62 (/26) | 192.168.2.0 | 255.255.255.192 | 192.168.2.1 | 192.168.2.62 | 192.168.2.63 |
| VLAN 20 Lab 2 & VLAN 20 Lab 1 | 40 | 62 (/26) | 192.168.2.64 | 255.255.255.192 | 192.168.2.65 | 192.168.2.126 | 192.168.2.127 |
| VLAN 30 | 35 | 62 (/26) | 192.168.2.128 | 255.255.255.192 | 192.168.2.129 | 192.168.2.198 | 192.168.2.191 |
| R5 to Main Campus Router Serial 0/0/0 | 2 | 2 (/30) | 192.168.100.4 | 255.255.255.252 | 192.168.100.5 | 192.168.100.6 | 192.168.100.7 |

### Mechanical Department

| Network Segment | Hosts Required | Hosts Available | Network Address | Subnet Mask | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VLAN 10 Admin | 50 | 62 (/26) | 192.168.3.0 | 255.255.255.192 | 192.168.3.1 | 192.168.3.62 | 192.168.3.63 |
| VLAN 20 Lab 2 & VLAN 20 Lab 1 | 40 | 62 (/26) | 192.168.3.64 | 255.255.255.192 | 192.168.3.65 | 192.168.3.126 | 192.168.3.127 |
| VLAN 30 | 35 | 62 (/26) | 192.168.3.128 | 255.255.255.192 | 192.168.3.129 | 192.168.3.198 | 192.168.3.191 |
| R6 to Main Campus Router Serial 0/0/0 | 2 | 2 (/30) | 192.168.100.8 | 255.255.255.252 | 192.168.100.9 | 192.168.100.10 | 192.168.100.11 |

## New Campus

### CS Department

| Network Segment | Hosts Required | Hosts Available | Network Address | Subnet Mask | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VLAN 10 Admin | 50 | 62 (/26) | 192.168.10.0 | 255.255.255.192 | 192.168.10.1 | 192.168.10.62 | 192.168.10.63 |
| VLAN 20 Lab 2 & VLAN 20 Lab 1 | 40 | 62 (/26) | 192.168.10.64 | 255.255.255.192 | 192.168.10.65 | 192.168.10.126 | 192.168.10.127 |
| VLAN 30 | 35 | 62 (/26) | 192.168.10.128 | 255.255.255.192 | 192.168.10.129 | 192.168.10.198 | 192.168.10.191 |
| R4 to Main Campus Router Serial 0/0/0 | 2 | 2 (/30) | 10.0.0.0 | 255.255.255.252 | 10.0.0.1 | 10.0.0.2 | 10.0.0.3 |

### Electrical Department

| Network Segment | Hosts Required | Hosts Available | Network Address | Subnet Mask | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VLAN 10 Admin | 50 | 62 (/26) | 192.168.20.0 | 255.255.255.192 | 192.168.20.1 | 192.168.20.62 | 192.168.20.63 |
| VLAN 20 Lab 2 & VLAN 20 Lab 1 | 40 | 62 (/26) | 192.168.20.64 | 255.255.255.192 | 192.168.20.65 | 192.168.20.126 | 192.168.20.127 |
| VLAN 30 | 35 | 62 (/26) | 192.168.20.128 | 255.255.255.192 | 192.168.20.129 | 192.168.20.198 | 192.168.20.191 |
| R4 to Main Campus Router Serial 0/0/0 | 2 | 2 (/30) | 10.0.0.4 | 255.255.255.252 | 10.0.0.5 | 10.0.0.6 | 10.0.0.7 |

### Mechanical Department

| Network Segment | Hosts Required | Hosts Available | Network Address | Subnet Mask | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VLAN 10 Admin | 50 | 62 (/26) | 192.168.30.0 | 255.255.255.192 | 192.168.30.1 | 192.168.30.62 | 192.168.30.63 |
| VLAN 20 Lab 2 & VLAN 20 Lab 1 | 40 | 62 (/26) | 192.168.30.64 | 255.255.255.192 | 192.168.30.65 | 192.168.30.126 | 192.168.30.127 |
| VLAN 30 | 35 | 62 (/26) | 192.168.30.128 | 255.255.255.192 | 192.168.30.129 | 192.168.30.198 | 192.168.30.191 |
| R4 to Main Campus Router Serial 0/0/0 | 2 | 2 (/30) | 10.0.0.8 | 255.255.255.252 | 10.0.0.9 | 10.0.0.10 | 10.0.0.11 |

## FSD Campus

### CS Department

| Network Segment | Hosts Required | Hosts Available | Network Address | Subnet Mask | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VLAN 10 Admin | 50 | 62 (/26) | 192.168.60.0 | 255.255.255.192 | 192.168.60.1 | 192.168.60.62 | 192.168.60.63 |
| VLAN 20 Lab 2 & VLAN 20 Lab 1 | 40 | 62 (/26) | 192.168.60.64 | 255.255.255.192 | 192.168.60.65 | 192.168.60.126 | 192.168.60.127 |
| VLAN 30 | 35 | 62 (/26) | 192.168.60.128 | 255.255.255.192 | 192.168.60.129 | 192.168.60.198 | 192.168.60.191 |
| R4 to Main Campus Router Serial 0/0/0 | 2 | 2 (/30) | 12.0.0.0 | 255.255.255.252 | 12.0.0.1 | 12.0.0.2 | 12.0.0.3 |

### Electrical Department

| Network Segment | Hosts Required | Hosts Available | Network Address | Subnet Mask | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VLAN 10 Admin | 50 | 62 (/26) | 192.168.70.0 | 255.255.255.192 | 192.168.70.1 | 192.168.70.62 | 192.168.70.63 |
| VLAN 20 Lab 2 & VLAN 20 Lab 1 | 40 | 62 (/26) | 192.168.70.64 | 255.255.255.192 | 192.168.70.65 | 192.168.70.126 | 192.168.70.127 |
| VLAN 30 | 35 | 62 (/26) | 192.168.70.128 | 255.255.255.192 | 192.168.70.129 | 192.168.70.198 | 192.168.70.191 |
| R4 to Main Campus Router Serial 0/0/0 | 2 | 2 (/30) | 12.0.0.4 | 255.255.255.252 | 12.0.0.5 | 12.0.0.6 | 12.0.0.7 |

### Mechanical Department

| Network Segment | Hosts Required | Hosts Available | Network Address | Subnet Mask | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VLAN 10 Admin | 50 | 62 (/26) | 192.168.80.0 | 255.255.255.192 | 192.168.80.1 | 192.168.80.62 | 192.168.80.63 |
| VLAN 20 Lab 2 & VLAN 20 Lab 1 | 40 | 62 (/26) | 192.168.80.64 | 255.255.255.192 | 192.168.80.65 | 192.168.80.126 | 192.168.80.127 |
| VLAN 30 | 35 | 62 (/26) | 192.168.80.128 | 255.255.255.192 | 192.168.80.129 | 192.168.80.198 | 192.168.80.191 |
| R4 to Main Campus Router Serial 0/0/0 | 2 | 2 (/30) | 12.0.0.8 | 255.255.255.252 | 12.0.0.9 | 12.0.0.10 | 12.0.0.11 |

# Key Configuration

## Switch Configuration

## Router Configuration

### Internal Routing

- OSPF used between:

- Internal routers

- Layer 3 switches

| Rule | Description |
| --- | --- |
| ACL1 | Block Admin (Sub-Campus 3) from accessing CS Web Server |
| ACL 2 | Allow all departments to access Email Server |
| ACL 3 | Restrict inter-department access where required |

## Router CLI

## Server Configuration

# Testing Results

## Pinging

### Ping Test (CS → EE)

## Trace Route
Trace route from one department to another department

# Packet Simulation

# Troubleshooting Commands Used

- ping

- tracert

- show ip route

- show vlan brief

- show ip ospf neighbor

- show ip bgp summary

# Redundancy And Failover Testing

- Border router shutdown tested

- Traffic successfully rerouted via alternate border router

- No loss of connectivity observed

# Conclusion

The designed network successfully meets all requirements of the CSC203L Open Ended Lab. The project demonstrates effective use of VLANs, VLSM-based IP addressing, dynamic routing protocols (OSPF & BGP), ACL-based security, and redundancy through multiple border routers. All test cases were validated, ensuring reliable and secure inter-campus communication.