University of Engineering and Technology, New Campus, Lahore

Department of Computer Science

Open Ended Lab

Course Code & Title: CSC203L Computer Networks Lab

1. Course Learning Outcome:

CLO 4: Design, configure, and test small computer networks using appropriate addressing schemes, protocols, and simulation tools.

Level of CLO: CLO3 is mapped to the 6th level, i.e., Creation of Bloom’s taxonomy.

2. Schwab/Herron Levels of Laboratory Openness

| Level | Methodology | Description | Problem | Ways & Means | Answers |
| --- | --- | --- | --- | --- | --- |
| 2 | Guided Inquiry | Students investigate a teacher presented question using student designed/selected procedures | Given | Open | Open |

3. Problem Statement:

Design and implement a university Network with multiple sub-campus each campus having multiple departments using a network simulator Cisco Packet Tracer. Each campus should include at least three departments (e.g., Computer Science, Electrical Engineering, and Administration) and a server room hosting core services. The project will require configuring various network devices, implementing VLANs, establishing inter-router routing, and testing end to end connectivity.

Problem Description:

### Network Design Requirements:

Network Topology:

- The network topology must be multi-campus (at least 2 sub-campus & one main campus), with a star-shaped design connecting each sub-campus to a central core switch, but also ensuring communication between different campus locations via border routers.

- Each sub-campus (i.e., the departments) should have its own border router for connectivity between sub-campuses and the central network.

- At least 4 border routers should be used: one for each sub-campus for high availability and redundancy, and inter-campus communication.

- The main campus can still have the central core switch, but each sub-campus should be interconnected with multiple border routers to ensure there’s no single point of failure in inter-campus communication.

Devices & Network Components:

Routers:

- At least 8 routers will be used:

  - 4 border routers, each located at one sub-campus for inter-campus communication and to ensure redundancy.

  - 4 internal routers (one for each sub-campus department) to handle routing between departments and internal communication.

Switches:

- 5 switches will be used, including one Layer 3 switch at each sub-campus for routing between VLANs and inter-campus routing.

End Devices:

- 10+ PCs per department, along with printers and mobile devices (e.g., Computer Science: 10 PCs, Electrical Engineering: 10 PCs, Administration: 10 PCs).

- Mobile Devices: Wireless devices (smartphones, laptops) will also be integrated into the network via access points at each sub-campus.

Servers:

- At least two servers per department: one for HTTP and another for Email.

- Two additional servers for public services such as a Web Server, DNS server, and FTP server for file sharing and storage across departments at each sub-campus.

ISP Connection:

- The network must be connected to an ISP using 4 border routers, one for each sub-campus, ensuring multiple paths to the internet and between sub-campuses for load balancing and failover.

### 2. Network Configuration:

IP Addressing & Subnetting:

IPv4 Addressing:

- The network will use IPv4 addressing.

- Each sub-campus (Computer Science, Electrical Engineering, and Administration) will have its own subnet with VLSM (Variable Length Subnet Masking) for efficient use of IPs.

- Each sub-campus will have its own public IP range provided by the ISP for direct internet connectivity.

- The private IP range for internal network communication (192.168.0.0/16) will be used across all sub-campuses.

Private IP Addressing Scheme:

- For the internal network, private IPs from 192.168.0.0/16 will be used for subnets.

- Example:

  - Main Campus could use 192.168.1.0/24

  - (Administrator in main campus)

  - Sub-Campus 1 (Having 3 different departments) could use 192.168.2.0/24

  - Sub-Campus 2 (Having 3 different department) could use 192.168.3.0/24

VLAN Configuration:

- Each department within each sub-campus will be placed in a separate VLAN for isolation and performance optimization.

  - VLAN 10: Computer Science

  - VLAN 20: Electrical Engineering

  - VLAN 30: Administration

- Wireless Devices: Separate VLANs for wireless devices at each sub-campus to segregate traffic and improve performance.

Inter-VLAN Routing:

- The Layer 3 switches will perform inter-VLAN routing between the different VLANs within each sub-campus and between sub-campuses.

Routing Protocols:

- OSPF (Open Shortest Path First) will be used for internal routing between the internal routers at each sub-campus and the core network.

- The border routers at each sub-campus will use BGP (Border Gateway Protocol) for inter-campus routing, ensuring load balancing and redundancy between sub-campuses.

Server Configuration:

- Web Server: A server for hosting web services (HTTP) for the entire campus network.

- Email Server: A server for handling email services (SMTP, POP3).

- DNS Server: A server for handling domain name resolution.

- FTP Server: A server for storing and sharing files between departments across all sub-campuses securely.

Security:

- Implement ACLs (Access Control Lists) to restrict access between the different departments, sub-campuses, and servers.

- The border routers will implement basic firewall rules to block unauthorized external access between sub-campuses and from the outside world.

### 3. Testing & Troubleshooting:

Test Cases:

1. Inter-Unit Communication:

- Test 1: Ensure Computer Science (Unit A) at Sub-Campus 1 can communicate with Electrical Engineering (Unit B) at Sub-Campus 2 and Administration (Unit C) at Sub-Campus 3.

- Test 2: Test communication between Sub-Campus 1 and Sub-Campus 2 (e.g., Computer Science to Electrical Engineering).

- Test 3: Check communication between end devices within each department at each sub-campus.

2. Server Access:

- Test 1: Verify that Computer Science (Unit A) at Sub-Campus 1 can access the Web Server and Email Server located in the central server room.

- Test 2: Ensure that Electrical Engineering (Unit B) at Sub-Campus 2 can access the DNS Server at the central server room.

- Test 3: Administration (Unit C) at Sub-Campus 3 should only be able to access the Email Server.

3. Security:

- Test 1: Ensure that Administration (Unit C) at Sub-Campus 3 cannot access the Web Server in Computer Science at Sub-Campus 1.

- Test 2: Use ping, tracert, and show commands to troubleshoot and verify routing functionality between different sub-campuses.

Troubleshooting:

- Use the show ip route command to verify routing tables and ensure proper routing between VLANs and sub-campuses.

- Troubleshoot VLANs and subnetting using show vlan brief to check VLAN configurations at each sub-campus.

- Verify that wireless devices at each sub-campus are able to connect, obtain IP addresses via DHCP, and access network services such as FTP, DNS, and HTTP.

4. Instructions:

- This is a group project; each group will consist of 4 students.

- Your project will be graded according to the provided rubrics.

- Proper network labeling (device names, IP addresses, VLAN IDs, etc.) is mandatory.

- Any form of plagiarism or copying of simulation files/configurations will result in penalties as per department policy.

- Deadline to submit: 5th January 2026

5. Deliverables:

Simulation File:

  - Complete Packet Tracer file (.pkt) with the network topology, configurations, and simulations.

  - The simulation file should reflect the updated network design with multiple border routers for inter-campus communication and redundancy.

Documentation:

  - Network Diagram: Clearly labelled topology diagram with IP addressing scheme, device names, and multiple border routers for each sub-campus.

  - VLAN Table: List of VLAN IDs and their corresponding department names, including separate VLANs for wireless devices.

  - IP Addressing Table: Detailed table of assigned IP ranges and subnets for the main campus and sub-campuses with multiple public IP ranges for each sub-campus.

  - Key Configuration Details:

    - Switch configurations (e.g., trunking, VLAN assignments).

    - Router configurations (e.g., OSPF, BGP, RIP for inter-campus routing).

    - Server configurations (e.g., IP settings for HTTP, Email, FTP, and DNS servers).

  - Testing Results:

    - Logs and screenshots from ping and tracert tests.

    - Results from server access tests, including restricted and allowed access based on ACLs and testing between sub-campuses.

  - Redundancy Testing: Ensure the border routers for each sub-campus function correctly with failover in case of a router failure

Open Ended Lab Assessment Rubrics

| Criteria | Excellent (7) | Good (5-6) | Satisfactory (3-4) | Needs Improvement (1-2) | Poor (0) |
| --- | --- | --- | --- | --- | --- |
| Network Design & Topology | Well-structured, clear, scalable design; appropriate choice of devices; logical separation of departments and sub-campuses; multiple border routers and inter-campus connections. | Mostly clear design; minor issues in device selection, scalability, or labeling. | Functional design but lacks clarity or scalability; some devices/links not clearly labeled. | Disorganized or unrealistic design; several missing/incorrect links or labels. | No coherent design; topology incomplete or irrelevant. |
| IP Addressing & VLAN Planning | Correct and efficient subnetting/VLSM; no overlaps; clear IP and VLAN tables; multiple public IPs for sub-campuses; VLANs properly defined for each department and wireless devices. | Mostly correct IP scheme; minor mistakes or inefficiencies; VLANs mostly appropriate. | Basic addressing works but may be inefficient; VLANs partially defined or not fully justified. | Major addressing or VLAN errors; connectivity only partially possible. | Addressing scheme invalid; VLANs missing or incorrect. |
| Configuration & Implementation | All required devices configured correctly (routers, switches, servers, border routers, wireless devices); network fully functional as per the problem statement. Includes multiple border routers for redundancy and load balancing. | Most configurations correct; minor issues with some services or routing but overall network largely functional. | Core configurations done; basic connectivity present; some services or features missing/incorrect. | Incomplete configurations; several parts of the network not working. | Little or no correct configuration; network not working. |
| Testing & Troubleshooting | Comprehensive testing; appropriate use of commands (ping, tracert, show, etc.); issues identified and correctly resolved with clear reasoning. Includes troubleshooting between sub-campuses and verification of border routers' functionality. | Adequate set of tests; most major issues found and resolved; minor gaps in troubleshooting detail. | Basic tests performed; at least one issue identified and addressed; limited justification. | Minimal testing; issues detected but not properly analyzed or resolved. | No meaningful testing; problems left unresolved. |
| Documentation & Presentation | Report is well-structured and clear; includes diagrams, tables, and explanations; configurations and results are easy to follow and professional. Updated to reflect sub-campus structure and multiple border routers. | Report mostly complete and understandable; minor formatting or clarity issues; most required elements included. | Documentation present but lacks detail or organization; some required elements missing. | Poorly organized or incomplete documentation; difficult to understand; major parts missing. | No proper documentation; work cannot be evaluated. |